CarbonVault™   Sentinel Rig Integration Brief

Daxem Labs   ·   Confidential   ·   February 2026   ·   Patent Filed GB2602946.2

WHAT YOU'RE BUILDING

The Sentinel Rig sits on each HGV and does three things:

—  Measure  fuel consumption directly from the fuel line using a flow sensor.

—  Read  engine data from the OBD-II port — RPM, speed, and engine load.

—  Transmit  a signed JSON packet to our backend every 30 seconds via MQTT over 4G.

That packet is the only input the platform needs. Everything else — the CO2 calculation, the carbon credit, the blockchain token — is handled automatically.

HOW THE DATA FLOWS

| Step 1 — Rig | Flow sensor reads fuel pulse. OBD-II reads RPM, speed, engine load. GPS provides the clock timestamp. |
| --- | --- |
| Step 2 — Sign | The rig hashes the full packet with SHA-256 and signs it. This is what makes the credits auditable — every credit traces back to a physical sensor reading. |
| Step 3 — Transmit | Packet sent via MQTT over 4G (Quectel BG96) to our broker, which relays it to the backend via WebSocket. |
| Step 4 — Platform | Backend validates, runs the CO₂ calculation, mints the credit. Frontend updates in real time. Nothing needed from you here. |

THE PACKET — WHAT TO TRANSMIT

Every packet must be valid JSON. Fields marked * are required — missing any of these will cause the packet to be rejected.

| Field | Type | Notes |
| --- | --- | --- |
| rigId * | string | Unique rig ID — assigned at provisioning e.g. SENT-001 |
| vehicleId * | string | Vehicle reg mapped in fleet config e.g. PK001 |
| timestamp * | number | Unix milliseconds — use GPS clock, NOT device clock (drifts) |
| fuelPulse_L * | float | Litres consumed since last packet — from flow sensor. This is the critical field. |
| hash * | string | SHA-256 of this packet, signed at source Format: sha256:abc123... |
| obdRpm | integer | Engine RPM — OBD-II PID 0x0C |
| obdSpeed_kmh | integer | Vehicle speed — OBD-II PID 0x0D |
| obdEngineLoad | float | Engine load 0.0–1.0 — OBD-II PID 0x04 |
| gpsLat | float | Latitude — WGS84 decimal degrees |
| gpsLng | float | Longitude — WGS84 decimal degrees |
| gpsAccuracy_m | float | GPS fix accuracy in metres |
| signalStrength | integer | 4G RSSI in dBm — Quectel BG96 |
| sdCardHash | string | SHA-256 of the local SD card log entry for this reading |
| firmwareVersion | string | Sentinel Rig firmware version e.g. 1.0.3 |

HOW THE PLATFORM USES YOUR DATA

Once the packet arrives, the following happens automatically. You don't need to implement any of this — just send the right numbers.

| CO₂ from fuel | fuelPulse_L × 2.68 = kg of CO₂ (DEFRA 2024 diesel factor) |
| --- | --- |
| Credit generated | CO₂ kg ÷ 1000 × 18% = tCO₂e credit (18% verified efficiency baseline) |
| Revenue | tCO₂e × £72/tonne × 70% = client’s annual share |
| Audit trail | Your packet hash + SD card hash + blockchain token = immutable chain of custody for Verra |

The only number that really matters from the rig is fuelPulse_L. Get that right and everything else follows.

TRANSMISSION SPEC

| Protocol | MQTT over TLS — port 8883 |
| --- | --- |
| Transport | 4G LTE — Quectel BG96 modem |
| Frequency | Every 30 seconds, or immediately on a significant fuel pulse event |
| Backend endpoint | wss://api.daxem.ai/rigs/live (WebSocket relay from MQTT broker) |
| Payload | JSON — one packet per message, no batching |
| Encryption | AES-256 in transit. Packet SHA-256 signed at source. |
| Offline | Buffer to SD card. Transmit catch-up batch when connectivity restored. |
| SD card | Write every reading locally regardless of connection. This is the audit black box. |

NOTES

—  Live demo:  The frontend is already built — jojibhatia.github.io/CarbonVault-Sandbox. Open it in a browser and you'll see exactly what happens when live packets start arriving. It runs in simulation mode until a real rig connects.

—  Developer Mode:  Press Ctrl+D in the dashboard to open a panel showing WebSocket status, the expected packet schema, and a live connection log. Useful for debugging during rig testing.

—  The hash matters:  Without a valid SHA-256 hash on each packet, Verra cannot verify the chain of custody from sensor to token. SHA-256 of the serialised packet before transmission is all that's needed.

—  GPS clock only:  Use the GPS module clock for timestamps, not the device clock. Device clocks drift. GPS is accurate to milliseconds and it matters for the audit trail.

Any questions on the spec — come back before building. Easier to fix on paper than in hardware.